# IP ADDRESSES

## Public vs Private IP Address

### Public IP

- Used on the internet

- Globally unique

- Assigned by ISP

- Identifies your network to the outside world
Example: 8.8.8.8

### Private IP

- Used inside local networks

- Not routable on the internet

- Reused across many networks
Ranges:

- 192.168.0.0 – 192.168.255.255

- 10.0.0.0 – 10.255.255.255

- 172.16.0.0 – 172.31.255.255
Example: 192.168.1.10

## Static vs Dynamic IP Address

### Static IP

- Does not change

- Manually assigned

- Used for servers
Example: Web server IP

### Dynamic IP

- Changes automatically

- Assigned by DHCP

- Used for home users
Example: Your home Wi-Fi IP

## How Devices Talk via Router (NAT)

- Devices have private IPs

- Router has one public IP

- Router uses NAT (Network Address Translation)

Flow:

Laptop (192.168.1.5) → Router → Internet

Internet → Router → Laptop

Router keeps a table to map internal IP + port ↔ public IP + port

# IP VERSIONS

## IPv4

- 32-bit address

- Written in decimals

- ~4.3 billion addresses
Example: 192.168.1.1

## IPv6

- 128-bit address

- Written in hex

- Huge address space
Example: 2001:0db8:85a3::8a2e:0370:7334

### Why IPv6?

- IPv4 addresses exhausted

- Better security & efficiency

- No need for NAT

# SUBNETTING

## What is Subnetting?

Dividing a network into smaller logical networks.

### Why Subnet?

- Better security

- Less traffic

- Efficient IP usage

### Example:

- Network: 192.168.1.0/24

- Subnets:

  - 192.168.1.0/26

  - 192.168.1.64/26

Subnet mask decides network part vs host part.

# ARP (Address Resolution Protocol)

## What is ARP?

ARP maps an IP address to a MAC address inside a local network.

## Why ARP is Needed

- Devices communicate using MAC addresses at Layer 2

- Applications use IP addresses at Layer 3

- ARP bridges IP → MAC

## How ARP Works

- Device broadcasts: “Who has 192.168.1.1?”

- Target replies with its MAC address

- Entry is stored in ARP table

## Example

192.168.1.1 → 00:1A:2B:3C:4D:5E

# DHCP (Dynamic Host Configuration Protocol)

## What is DHCP?

Automatically assigns IP configuration to devices.

## What DHCP Provides

- IP address

- Subnet mask

- Default gateway

- DNS server

## DHCP Process (DORA)

- Discover – Client looks for DHCP server

- Offer – Server offers IP

- Request – Client requests IP

- Acknowledge – Server confirms

## Example

Laptop joins Wi-Fi → gets 192.168.1.25

# NAT (Network Address Translation)

## What is NAT?

NAT allows multiple private IPs to access the internet using one public IP.

## Why NAT is Used

- Saves public IP addresses

- Adds basic security

- Enables private networks

## Types of NAT

- Static NAT: One private ↔ one public IP

- Dynamic NAT: Pool of public IPs

- PAT (Port Address Translation): Many private IPs → one public IP using ports

# NAT Table

## What is a NAT Table?

A table in the router that keeps track of private IP + port ↔ public IP + port

## Example NAT Table Entry

| Private IP | Private Port | Public IP | Public Port |
| --- | --- | --- | --- |
| 192.168.1.10 | 52344 | 203.0.113.5 | 40001 |

## Why NAT Table is Important

- Routes responses back to correct device

- Enables multiple devices to share one public IP

# ARP vs DHCP vs NAT (Quick Difference)

| Feature | ARP | DHCP | NAT |
| --- | --- | --- | --- |
| Purpose | IP → MAC | Assign IP | Private ↔ Public IP |
| Layer | Data Link | Application | Network |
| Used Where | Local network | Local network | Router / Gateway |

## One-Line Flow Example (All Together)

- DHCP gives IP to laptop

- ARP finds MAC of router

- NAT translates private IP to public IP

- NAT table tracks the connection

# PORTS

## What is a Port?

- A logical number that identifies a service on a device

- Allows multiple services on one IP

IP = house address
Port = room number

## Types of Ports

### Well-Known Ports (0–1023)

Used by common services

- HTTP → 80

- HTTPS → 443

- FTP → 21

- SSH → 22

### Registered Ports (1024–49151)

Used by applications

- MySQL → 3306

### Dynamic / Ephemeral Ports (49152–65535)

Temporary ports for clients

- Browser connecting to a website

## Example:

192.168.1.10:443

IP = device
Port = HTTPS service

# PROTOCOLS

## What is a Protocol?

A set of rules that defines how data is communicated.

### Common Protocols

- HTTP → web pages

- HTTPS → secure web

- FTP → file transfer

- SMTP → sending emails

- DNS → name to IP resolution

- TCP → reliable transport

- UDP → fast transport

## Port vs Protocol (Difference)

| Port | Protocol |
| --- | --- |
| Number | Rule |
| Identifies service | Defines communication method |
| Example: 80 | Example: HTTP |

👉 Protocol decides HOW, port decides WHERE

## Why TCP/IP Has Ports but TCP/IP Itself Doesn’t Use a Port

- Ports belong to Transport layer (TCP/UDP)

- TCP/IP is a model, not a service

- TCP/IP defines how data moves, not a service endpoint

Example:

- HTTP uses TCP port 80

- TCP itself does not use a port

# OSI MODEL (7 Layers)

From bottom → top:

## 1. Physical

- Sends raw bits (cables, signals)

- Example: Ethernet cable

## 2. Data Link

- MAC addressing

- Error detection

- Switches work here
Example: MAC address

## 3. Network

- Logical addressing & routing

- IP addresses

- Routers work here
Example: IPv4, IPv6

## 4. Transport

- End-to-end delivery

- Reliability & ports
Example: TCP, UDP

## 5. Session

- Manages sessions

- Open, maintain, close
Example: Login session

## 6. Presentation

- Data format, encryption

- Compression
Example: SSL/TLS

## 7. Application

- User-facing services
Example: HTTP, FTP, SMTP

# TCP/IP MODEL (4 Layers)

| TCP/IP | OSI |
| --- | --- |
| Application | Application + Presentation + Session |
| Transport | Transport |
| Internet | Network |
| Network Access | Data Link + Physical |

## Key Difference: OSI vs TCP/IP

### OSI Model

- Conceptual

- Used for learning & troubleshooting

- 7 layers

### TCP/IP Model

- Practical

- Used in real networks

- 4 layers

👉 OSI explains how things should work
👉 TCP/IP explains how they actually work