# ARP (Address Resolution Protocol)

Layer: Data Link / Network (bridges Layer 2 and Layer 3)
Purpose: Maps IP addresses (Layer 3) to MAC addresses (Layer 2) so that devices on a local network can communicate.

## How ARP Works

- Device wants to communicate:

  - A computer wants to send a packet to an IP on the same local network.

  - It knows the IP address but not the MAC address.

- ARP Request (Broadcast):

  - The device broadcasts a message to all devices on the LAN:

- Who has IP 192.168.1.5? Tell 192.168.1.2

- ARP Reply (Unicast):

  - The device with the matching IP replies directly with its MAC address.

- Communication:

  - The sending device now encapsulates the IP packet inside a frame with the correct MAC address.

Notes:

- ARP is local network only.

- ARP entries are cached temporarily to reduce broadcasts.

- Tools like arp -a (Windows) or arp (Linux) can view the ARP table.

# NAT (Network Address Translation)

Layer: Network (Layer 3)
Purpose: Allows multiple devices on a private network to share a single public IP address for Internet access.

## How NAT Works

- Private IP Addresses:

  - Devices inside a LAN have private IPs (e.g., 192.168.x.x).

  - Private IPs cannot be routed over the Internet.

- Translation by Router:

  - When a device sends a packet to the Internet, the router replaces the source private IP with the router’s public IP.

  - Adds a unique port number to track which internal device sent the request (PAT / Port Address Translation).

- Return Traffic:

  - When the response comes back, the router uses the port mapping table to forward the packet to the correct internal device.

## Types of NAT

| Type | Description |
| --- | --- |
| Static NAT | Maps one private IP to one public IP permanently. |
| Dynamic NAT | Maps private IPs to a pool of public IPs temporarily. |
| PAT / Overloading | Many private IPs share a single public IP using unique ports. |

## Key Notes

- ARP: Used for MAC discovery on local network; resolves IP → MAC.

- NAT: Used to connect private networks to the Internet; resolves private IP → public IP and tracks ports.

- ARP works inside LAN, NAT works at the router level for Internet communication.